3/19/2023

Michael Wyatt

Design and Implementation of Chimera

# Introduction

Chimera is an advanced UNIX-like operating system written in Rust/Kotlin. Like the BSDs and Linux, low-level system functions such as device drivers and the virtual file system are executed in a shared address space, but with a twist: each component is a separate process communicating via message passing. This allows for modularity without the performance sacrifice found within most microkernel operating system designs, while also not sacrificing stability due to Rust’s safety features.

Most of the userspace applications are written in Kotlin and individually sandboxed inside of a virtual machine within the shared address space. Userspace processes that do not target the JVM instead utilize traditional hardware protection and execute in their own virtual address space. This method of combining Software Isolated Processes (SIPs) with Hardware Isolated Processes (HIPs) is referred to as the hybrid memory protection model. This design increases the portability of common apps such as the shell and system utilities without sacrificing the native bare metal performance required by games and complex computation software.

# 1.1 System Design Overview

Chimera utilizes a very small kernel to handle low level tasks such as handling interrupts, direct I/O access, facilitating scheduling/memory management, etc. All other functions are provided by separate processes called servers and drivers. When we say “the system” we are referring to these components collectively functioning as a unit. The components are listed below:

Kernel
Designed to be as minimal as possible, the kernels’ responsibilities are to manage processes, scheduling, and memory (at the hardware level), perform I/O operations for the device drivers, and completing system calls by facilitating message passing between the servers and drivers.

Servers
The remaining functionality expected of a typical UNIX-like operating system is implemented via a collection of server processes. Each server has a well-defined purpose and uses message passing to support the higher-level system concepts like forking a process or deciding when a process should be scheduled next and for how long.

Device Drivers
Device drivers are written in safe Rust and utilize Chimera’s driver API to manipulate hardware devices. The I/O handler has a granular access control policy to prevent drivers from accessing ports and memory regions that were not requested when they registered with the handler.

Any server/driver can be executed as a userspace process within its own virtual address space. This is useful both for development purposes and allowing the user to sandbox a server/driver they suspect may be problematic. It should be noted that besides the kernel and I/O handler, no other process on the system executes with access to privileged instructions. It is simple and efficient to change runlevels so long as a full address space change does not take place.

Userspace libraries/software interact with the system via traditional system call interrupts; the userspace need not be concerned with the inner workings of the system such as which server handles reading from the hard disk to open a file.

In the above example of opening a file, the software would utilize the open() system call. The implementation of the open() system call would place the files path in a register and use an interrupt to request service from the system. The interrupt service routine will pack the file path and other information into a message and place it on the VFS servers’ queue. The VFS server will communicate with the hard disk driver to read the file from the disk and send the bytes back through the kernel to the calling process.

# 1.2 Java Virtual Machine

Much of the system is written in Kotlin, necessitating the need for a powerful implementation of the Java Virtual Machine. While Chimera is still in its infancy, it will utilize a very simple JIT based VM like Oracles Hotspot VM. The long-term plan is to implement an AOT design similar to GraalVM to achieve performance closer to that of native code. This is a minor implementation detail and likely not something the user would likely be able to differentiate between in regular usage.

Since the VM process shares address space with the kernel and other sensitive processes/data structures it is crucial that the implementation is stable and secure. For this reason, the VM must be written specifically for Chimera in a memory safe language (Rust) as opposed to C with this concept in mind.

When a process is created the process server forks a new VM and loads the bytecode into memory. The VM checks the bytecode for errors and translates the bytecode into machine instructions. To protect against exploits, the VM contains no privileged code and is incapable of doing anything a userspace process in a traditional system would be incapable of doing. Direct memory access, I/O operations, IPC, etc. are all delegated to the system.

It is not possible to exploit the system by attempting to execute native code. Only the kernel and the VM can be executed on bare metal, and both must have a valid code signature. The entire standard library is written in Kotlin. When system access is needed the library utilizes ___SYSCALL__(): a custom syntax directive that compiles down to a VM opcode that fires interrupt 0x80 to perform system calls.

Processes that run within the VM are referred to as Software Isolated Processes (SIPs).

# 1.3 Native Application Software

The primary deterrent to large scale adoption of single address space operating systems is their lack of compatibility with the massive collection of application software that has been written over the last 40 years. Billions of dollars and countless hours were put into the development of this software; leaving this work behind is not an option.

It is for this reason that Chimera provides the necessary runtime environment for software that compiles down to native machine code. The protection model is no different than that of any other mainstream OS; the hardware provides the process its own virtual address space and prevents it from interfering with other processes on the system.

Processes that run directly on the CPU without emulation are referred to as Hardware Isolated Processes (HIPs).

# 2.0 System Specifications

The following chapters provide a deeper look at the internal design of Chimera.

# 2.1 Servers and their Functions

| Name | Description | Notes |
| --- | --- | --- |
| proc | Process Management | First process started after kernel. PID of 1. |
| crash | Crash Server | All servers/drivers are child processes of this server |
| sched | Scheduler |  |
| vfs | Virtual File System |  |
| vmem | Virtual Memory Manager |  |

# Roadmap

v0.1 – Bootstrap the Kernel

Get as far as you can go without having to load another process??